# REPORT FOR VAMAN BOARD SETUP IN TERMUX

(RUGGED PHONE)

Vemireddy Uma Gayathri

### Prepare android device:

- Enable developer options

### Go to settings → About phone → Find Build number

### Tap in 2-3 Times and you will see “you are developer”

### Enable USB Debugging

### Go to setting → Developer Options

### Turn on these toggles:

- USB debugging

- USB debugging (security settings)

- Connected to PC

- Plug in USB cable

- Tap “Allow USB Debugging”

- Checked always allow from this computer →tap ok

### Termux installation

- Installed Termux

https://www.f-droid.org/

Downloaded Termux from F-Droid (not Play Store for full functionality)

- After installation,I gave termux access to my android storage

termux-setup-storage

- Installed ubuntu inside termux

Install ubuntu (25.10), Because ubuntu 20 and 24 were not listed in proot-distro.

proot-distro install ubuntu (25.10 version)

- Login to ubuntu

proot-distro login ubuntu

- Then I updated and upgraded the system packages

apt update && apt upgrade

apt install build-essential

- Installed essential development tools

apt install git wget curl python3 python3-pip python3-venv -y

Setup pygmy-dev:

- Created a directory for building the toolchain

mkdir -p ~/symbiflow

export INSTALL_DIR=~/symbiflow

- Downloaded and extract symbiflow OR copy from another source

Latest: curl $(curl https://storage.googleapis.com/symbiflow-arch-defs-install/latest) > ~/arch.tar.gz

- But Get the arch.tar.gz from https://drive.google.com/file/d/17gVGRJ1qcaWanYzyg1eMUiLlFqya3_ZD to ~/arch.tar.gz

tar -C $INSTALL_DIR -xvf ~/arch.tar.gz

- Exported the file path

export PATH="$INSTALL_DIR/bin:$INSTALL_DIR/quicklogic-arch-defs/bin:$INSTALL_DIR/quicklogic-arch-defs/bin/python:$PATH"

- Cloned the Pygmy repo to local

cd ~

git clone --recursive https://github.com/optimuslogic/pygmy-dev

- Compiled yosys

cd ~/pygmy-dev/tools/quicklogic-yosys/

make config-gcc

make -j4 install PREFIX=$INSTALL_DIR

cd ~

- Compiled yosys-plugins

cd ~/pygmy-dev/tools/yosys-symbiflow-plugins/

export PATH=$INSTALL_DIR/bin:$PATH

make -j4 install

cd ~

- Compile VTR

cd ~/pygmy-dev/tools/vtr-verilog-to-routing/

- Using make -j1,because make -j4 is showing that signal 9 means that android kills heavy process.

- While Installing, I faced some errors related to including header files. So  I added header files(#include <limits>, #include <cstdint>)

- Rebuild make -j1. It took 30 to 40 minutes to installed

- Then changed the directory cd ~

- Copied the required binaries

cp ~/pygmy-dev/tools/vtr-verilog-to-routing/build/vpr/vpr    $INSTALL_DIR/bin

cp ~/pygmy-dev/tools/vtr-verilog-to-routing/build/utils/fasm/genfasm $INSTALL_DIR/bin

- Verified the tools

vpr -h

yosys -h

genfasm -h

- Install pip3 and python3 setup.py

cd ~/pygmy-dev/tools/quicklogic-fasm/

pip3 install -r requirements.txt

- While installing python requirements, I encountered an error because git://links were blocked

-e git://github.com/symbiflow/fasm.git#egg=fasm

-e git://github.com/antmicro/quicklogic-fasm-utils.git#egg=fasm-utils

To

-e git+https://github.com/symbiflow/fasm.git#egg=fasm

-e git+https://github.com/antmicro/quicklogic-fasm-utils.git#egg=fasm-utils

- Then ran the python3 setup.py install

cd ~

- Checked the tools

- qlfasm -h

- ql_symbiflow -h     -- it showed command is not found, So I tried this commands

pip3 Install gdown

gdown 17gVGRJ1qcaWanYzyg1eMUiLlFqya3_ZD -0 ~/arch.tar.gz

tar -C $INSTALL_DIR -xvf ~/arch.tar.gz

export PATH="$INSTALL_DIR/bin:$INSTALL_DIR/quicklogic-arch-defs/bin:$INSTALL_DIR/quicklogic-arch-defs/bin/python:$PATH"

Then tried  ql_symbiflow -h

- Tested FPGA compilation which uses the tools

cd $INSTALL_DIR/quicklogic-arch-defs/tests/counter_16bit

ql_symbiflow -compile -d ql-eos-s3 -P pd64 -v counter_16bit.v -t top -p  m chandalar.pcf

- Changed the directory

cd ~/pygmy-dev/pygmy-sdk/S7seg-apps/static-number/

ql_symbiflow -compile -src . -d ql-eos-s3 -t top -v seg7_static.v -p pygmy.pcf -P PU64 -dump binary

- Downloaded and extracted the ARM GCC toolchain

cd $INSTALL_DIR

wget https://developer.arm.com/-/media/Files/downloads/gnu-rm/9-2020q2/gcc-arm-none-eabi-9-2020-q2-update-aarch64-linux.tar.bz2

tar -xvjf  gcc-arm-none-eabi-9-2020-q2-update-aarch64-linux.tar.bz2

export PATH=$INSTALL_DIR/gcc-arm-none-eabi-9-2020-q2 update/bin:$PATH

- Tested  ARM gcc

arm-none-eabi-gcc –version

- Added the  ~/.bash_profile to setup the environment variables next time

export INSTALL_DIR=~/symbiflow

export PATH="$INSTALL_DIR/bin:$INSTALL_DIR/quicklogic-arch-defs/bin:$INSTALL_DIR/quicklogic-arch-defs/bin/python:$PATH"

export PATH=$INSTALL_DIR/gcc-arm-none-eabi-9-2020-q2-update/bin:$PATH

- Tested Software & FPGA code

- cd  ~/pygmy-dev/pygmy-sdk/qf_apps/qf_helloworldsw/GCC_Project/

make

- cd ~/pygmy-dev/pygmy-sdk/qf_apps/qf_helloworldhw/GCC_Project/

make

- I modified the helloworldfpga.v file and rebuild the project. The .bin file was generated successfully .

- I transferred the .bin file to my laptop and connected the vaman board. The flashing process completed successfully without errors.

- The .bin file(100kb) was generated successfully and shared to laptop (ubuntu 20 version)

- Finally the phone-generated .bin file was flashed and the LEDS blinked